Cambridge Guidelines

Appendix: Substitute materials

Draft

April 9, 2018

Note:  Below is a list of possible substitute materials. They should only be considered when the using the original material for repair or replacement is not technically or economically feasible. When selecting a substitute material, particular attention should be paid to the material's size, shape, texture, color, expansion and contraction rates, weathering, and chemical properties.  Other substitute materials may be considered by the HPC.

Material		Substitute Material

Built up roofing	Single ply roofing Elastomeric roofing

Canvas duck		Canvas nylon

Cast stone		Glass reinforced epoxy cement (GFRC)

Cedar or wood           Architectural Roofing Shingles

Roofing shakes

Clear glass		Clear E-glass

Columns

(Porch and house)       Fiberglass load bearing that match the profile and texture of the original

Concrete		Glass reinforced epoxy cement (GFRC)

Metal (tin or zinc)	Cast aluminum

Ornamentation	Fiber reinforced polymers

Epoxy or polymer concrete (polyurethane or polypropylene).

Slate roofing		Composite (fiberglass) or Rubber  slate if it matches the color, profile and

Dimensions of existing roof slates

Steel sash		Aluminum sash (baked enamel or powder coated finish)

Stone			Glass reinforced epoxy cement (GFRC)

Stucco			Acrylic polymer stucco

Terne plate                 Modern Terne II plate Lead or zinc coated cooper

Wood doors		Fiberglass doors

Metal doors (powder coated or baked enamel)

Wood ornamentation	Fiber reinforced polymers

Epoxy or polymer concrete (polyurethane or polypropylene).

Wood Porch

T&G flooring         Composite T&G flooring that closely matches the profile and width of

Of the original porch flooring

Wood shingles/

Shakes		Synthetic Vinyl molded shingles/shakes

Wood siding		Cementitious Siding, Vinyl Siding

Wood windows	Aluminum (baked enamel or powdercoated finish), vinyl or fiberglass clad

wood frame